Steps for How to build a circuit in Proteus 8.5

Phase 1: Component Gathering

- Before building, you must "pick" the parts from the library into your device list.

- Open the Library: Click the 'P' button (Pick Devices) in the Devices selector on the left toolbar.

Search Keywords:

- Type VSOURCE to find the DC voltage source.

- Type RES to find the generic resistor.

- Terminals: Click the Terminals Mode icon (two horizontal lines) and select GROUND. Every simulation in Proteus requires a ground for a zero-voltage reference.

Phase 2: Constructing Circuit 1 (3-Resistor Parallel)

- Place the Source: Select VSOURCE and click on the schematic to place it. Double-click the component and change the Voltage value to 9V.

- Place Resistors: Place three RES components vertically side-by-side to the right of the source.

- Set Resistance: Double-click each to set the values: 10k, 2k, and 1k.

- Wiring: Use the pencil tool (appears when hovering over pins) to connect the top of all resistors to the positive (+) terminal of the source and the bottom of all resistors to the negative (-) terminal.

- Add Ground: Attach the GROUND terminal to the negative side of the circuit.

Phase 3: Measuring Current and Voltage

To fill your "Practical Values" table, you must integrate virtual meters.

- Instrument Selection: Click the Virtual Instruments Mode icon (looks like a small multimeter) and select DC AMMETER and DC VOLTMETER.

- Measure Voltage (Nodes 2 & 7): Connect the DC Voltmeter in parallel across the source or any resistor branch.

- Measure Individual Currents: * Break the circuit: Delete the wire connecting the top of $R_1$ to the main rail.

- Insert Ammeter: Place the DC Ammeter in that gap so the current must pass through it to reach the resistor.

- Repeat: Do this for $R_2$ and $R_3$.

- Change Scale: Since our calculations are in milliamps, right-click each Ammeter $\rightarrow$ Edit Properties $\rightarrow$ change Display Range to Milliamps.

Phase 4: Constructing Circuit 2 (4-Resistor Node)

- Repeat Placement: Place a 9V VSOURCE and four resistors.

- Update Values: Set them to 100 (no 'k'), 3k, 5k, and 10k.

- Node Connection: Connect all left-side terminals to a single point (Node a) and all right-side terminals to another point (Node e).

- Finalize: Connect the source across these two points. Insert ammeters in each of the four branches using the same "series" method from Phase 3.

Phase 5: Running the Simulation

- Start: Click the Play button in the bottom-left animation control bar.

- Observe: The meters will now display the simulated values

- Record: Write these values into your "Practical Values" table. In a simulation, these should match your Theoretical Values ($0.9\text{mA}, 4.5\text{mA}$, etc.) almost perfectly.

- Stop: Click the Square (Stop) button before making any further wiring changes.